Quotation for Renovation of CRIDA Canteen at Main Office complex.

| Sl.no. | Description of item | Unit | Qty. | Rate | Amount(`) |
| --- | --- | --- | --- | --- | --- |
| 1 | Dismantling stone slab flooring laid in cement mortar including stacking of serviceable material and disposal of unserviceable material within 50 metres lead. 1x10.00x10.00 = 100.00 sqm | sqm | 100.00 |  |  |
| 2 2.1 | Dismantling tile work in floors and roofs laid in cement mortar including stacking material within 50 metres lead. For thickness of tiles 10 mm to 25 mm 1x4.70x2.80 = 13.16 sqm | sqm | 13.16 |  |  |
| 3 | Providing & laying vitrified floor tiles in different sizes(thickness to be specified by the manufacturer)with water absorption less than 0.08% & conforming to IS: 15622, of approved make, in all colours & shades, laid on 20mm thick cement mortar 1:4(1 cement : 4 coarse sand), including grouting the joints with white cement & matching pigments etc., complete. Size of Tile 600x600 mm 1x10.00x10.00 = 100.00 sqm | sqm | 100.00 |  |  |
| 4 | Providing & laying Ceramic glazed floor tiles of size 300x300 mm(thickness to be specified by the manufacturer) of 1st quality conforming to IS : 15622 of approved make in colours such as White, Ivory, Grey, Fume Red Brown, laid on 20 mm thick cement mortar 1:4 (1 Cement : 4 Coarse sand), including pointing the joints with white cement and matching pigment etc., complete. 1x4.70x2.80 = 13.16 sqm | sqm | 13.16 |  |  |
| 5 | Removing white or colour wash by scrapping & sand papering & preparing the surface smooth including necessary repairs to scratches etc. complete. Walls:4x10.00x3.30 = 132.00 2x4.70x3.30 = 31.02 3x2.80x3.30 = 27.72 Ceiling:1x10.00x10.00= 100.00 1x4.70x3.30 = 15.51 Total = 306.25 sqm | sqm | 306.25 |  |  |
| 6 | Distempering with oil bound washable distemper of approved brand and manufacture to give an even shade : Old work (one or more coats) Qty. same as Sl.no. 5 = 306.25 sqm | sqm | 306.25 |  |  |
| 7 | Removing & Refixing of Aluminium doors | each | 6 nos. |  |  |
| 8 | Disposal of debris & other unserviceable materials out of campus as directed by site Engineer. |  | One Job |  |  |

Total `

Signature of the Contractor

Contact No: